المتطلبات الوظيفية لنظام الصوت والصورة

1.  عرض المحتوى على شاشة تفاعلية من مصادر متعددة:

أ. جهاز حاسوب عبر كابل HDMI أو لاسلكيًا عبر. Air Media

ب. كاميرا PTZ.

جـ. عرض بث كاميرا مثبتة على درون لعرض مشاهد حية.

د. عرض المحتوى من منصات الاجتماعات مثل Zoom أو .Teams

2.  نظام ميكروفونات:

أ. عدد 19 ميكروفون Delegate للمشاركين في الجلسة.

ب. عدد 1 ميكروفون Chairman للمسؤول أو المتحدث الرئيسي.

3.  ميكروفون لاسلكي Handheld: توفير ميكروفون لاسلكي Handheld واحد لاستخدام المتحدثين المتنقلين.

4.  سماعات: توفير سماعات مناسبة لتوزيع الصوت في كافة أنحاء الغرفة، بحيث تكون عدد السماعات كافيًا لتغطية الغرفة بشكل كامل وتوزيع الصوت بشكل متوازن.

5.  شاشة لمس (Touch Screen):

أ. توفير شاشة لمس للتحكم في الشاشة التفاعلية، بحيث تسمح بالتبديل بين مصادر الفيديو والتحكم في مستوى الصوت

ب. ربط شاشة اللمس بـ جهاز تحكم مركزي لضبط وإدارة المصادر المعروضة.

6.  جهاز تحكم بالصوت: توفير وحدة (DSP) للتحكم في مستوى الصوت لكل مصدر، مع إمكانية التحكم في تقليل الضوضاء (Noise Reduction) وتعديل المستويات الصوتية بشكل ديناميكي.

8.  جهاز Multi viewer: توفير جهاز Multi viewer لعرض عدة مصادر على الشاشة التفاعلية في نفس الوقت، بما في ذلك الكاميرات المختلفة، محتوى الحاسوب، ومحتوى الاجتماعات من Zoom أو. Teams

7.  جهاز Amplifier: توفير جهاز Amplifier لضمان تعزيز الصوت وتوزيعه بشكل متوازن داخل الغرفة. يجب أن يكون هناك قدرة على التحكم في مستويات الصوت حسب الحاجة.

8.  متطلبات التوزيع والتهيئة في الغرفة:

أ. الغرفة ذات أبعاد طول 11.5 متر، عرض 6.7 متر، ارتفاع 3.25 متر.

ب. توزيع السماعات بشكل مناسب لضمان تغطية الصوت بشكل متساوي ونقي بدون صدى.

جـ. تركيب الأجهزة بشكل يسهل الوصول والتحكم في جميع الأجهزة من خلال شاشة اللمس.

| QTY | المواصفات الفنية | ITEM | NO. |
| --- | --- | --- | --- |
| 1 | - Screen Size: 98 inches (minimum) - Backlight Type: Direct LED (D-LED) - Resolution: 3840 × 2160 (4K UHD) - Brightness: ≥ 450 cd/m² (typical) - Contrast Ratio: ≥ 1200:1 (typical) - Response Time: ≤ 8 ms - Pixel Pitch: 0.56 mm (H) × 0.56 mm (V) - Refresh Rate: 60 Hz - Viewing Angle: ≥ 170° (H) / 170° (V) - Life Expectancy: ≥ 50,000 hours (minimum) - Touch Points: ≥ 20 simultaneous touch points - Connectivity: HDMI, USB, Display Port, LAN, Audio In/Out - Mounting: Wall-mounted or mobile trolley (as required) | Interactive Display Panel | 1 |
| 1 | - Input Ports: 2 × HDMI (19-pin Type-A), 2 × USB-C - Output Ports: 2 × HDMI (19-pin Type-A), 1 × Analog Audio Out - USB-C Power Delivery: ≥ 60 W PD - Supported Functions: Matrix Output, Multiview Display, and Wireless Casting - Wireless Casting: AirPlay and Chromecast compatible - Supported Resolution: Up to 4K@60Hz | Multi-Viewer 4K Presentation Switcher | 2 |
| 1 | - Type: USB-C Wireless Presentation Dongle - Compatibility: Windows, macOS, Android, iOS - Transmission: Full HD (1080p) minimum - Security: Encrypted connection with PIN pairing | Wireless Casting Dongle | 3 |
| 2 | - Design: Compact in-desk connectivity solution - Ports: HDMI x1, USB charging x1 - Power: Integrated AC power outlet - Build: Durable high-grade housing | Cable Management Module (Cable Caddy) | 4 |
| 1 | - Communication Interfaces: Ethernet, USB, RS-232/422/485, IR, Relay - Memory: Minimum 1 GB SDRAM, 8 GB Flash - External Storage: Supports USB mass storage - Supported Features: Device automation, power scheduling, input selection - Connectors: LAN (PoE supported), USB-OTG, RELAY (1–2), IR (1–2), COM - Integration: Compatible with major AV control protocols (AMX / Crestron / Extron or equivalent) | Room Controller | 5 |
| 1 | - Display Type: LCD / IPS capacitive touch screen - Brightness: ≥ 350 nits (cd/m²) - Contrast Ratio: ≥ 850:1 - Mounting: Desktop / table-mounted - Function: Full control of AV system | Tabletop Touch Screen | 6 |
| 6 | - Power Rating: 30 W (100 V line) - Sensitivity: ≥ 90 dB @ 1 W / 1 m - Frequency Response: 80 Hz – 16 kHz (±3 dB) - Tap Settings: 30 W to 5 W selectable - Speaker Type: 2-way wall-mountable with metal grille. | Wall Speakers | 7 |
| 1 | - Rated Output Power: ≥ 240 W @ 100 V line - Frequency Response: 50 Hz – 20 kHz (±1 dB) - Total Harmonic Distortion: < 1% @ rated power - Signal-to-Noise Ratio: ≥ 60 dB - Compatible with 100 V distributed audio systems | Power Amplifier | 8 |
| 1 | - Type: 6×4 ProDSP Digital Matrix Processor - Inputs/Outputs: 6 mic/line inputs, 4 line outputs - Acoustic Echo Cancellation (AEC): Built-in, 6 channels minimum - Connectivity: Configurable USB audio interface for integration - Sampling Rate: ≥ 48 kHz, 24-bit audio processing | DSP with Echo Canceller | 9 |
| 1 | - Function: Central controller of the conference system - Capacity: Supports up to 246 delegate/chairman units - Installation: Rack-mountable, easy setup - Features: - Adjustable number of open microphones - Auto mic-off function - Speech priority and feedback suppression - Integrated USB audio interface - Mode selection for ID assignment - Interfaces: RS-232, Ethernet, USB | Master Control Unit (CU) | 10 |
| 1 | 1. Operation: Voice-activated or manual mode 2. Audio: Built-in speaker and microphone 3. Controls: Priority button with override function 4. Headphone Jack: 3.5 mm, with individual volume control 5. Build: Compact and ergonomic design | Chairman Unit | 11 |
| 19 | - Operation: Voice-activated or manual mode - Audio: Built-in loudspeaker and microphone - Headphone Jack: 3.5 mm, with individual volume control - System Compatibility: Master CU and Chairman Unit - Easy plug-and-play installation via daisy-chain cabling | Delegate Units | 12 |
| 20 | - Type: Unidirectional electret condenser microphone - Connector: XLR-3-32 - Sensitivity: –40 dB (1 kHz, 0 dB = 1 V/Pa) - Frequency Response: 100 Hz – 13 kHz - Length: ≥ 500 mm - Features: Illuminated red ring to indicate active microphone | Gooseneck Microphones | 13 |
| 1 | - Type: UHF or 2.4 GHz digital wireless system - Range: ≥ 50 meters (line-of-sight) - Audio Frequency Response: 50 Hz – 15 kHz - Battery: Rechargeable lithium or AA type | Wireless Handheld Microphone Set | 14 |
| 1 | - Function: auto-tracking, audio and video capture - Video Resolution: 1920×1080p @ 60 Hz (Full HD) - Optical Zoom: Minimum 12× - Digital Zoom: Minimum 15× - Output Ports: USB 3.0 Type-C, RJ-45 Ethernet, HDMI Type-A - Control Protocols: VISCA / Pelco-D / Pelco-P / ONVIF - Preset Positions: ≥ 10 programmable - Pan / Tilt Range: Pan: ±170°, Tilt: –30° to +90° - IR receiver, RS-232, and Ethernet control support | PTZ Camera | 15 |
| ls | - Structured cabling using CAT6 UTP/FTP | Communication and Accessories | 16 |
| 1 | - 24 U with power distribution, fans, and all accessories | RACK CABINET | 17 |
| - Supplier shall perform complete system configuration, testing, and commissioning. - Verification to include: - Audio clarity and AEC performance - Camera tracking accuracy - Control integration functionality - Full documentation and training for end users - All equipment must be CE, FCC, and RoHS compliant. - Manufacturer shall be ISO 9001 certified. - All components must be new, genuine, and from the same system family for full compatibility. - Installation to be carried out by qualified and authorized personnel. |  | Testing and Commissioning | 18 |

الشروط الخاصة

1. تلتزم الشركة بتوريد الأجهزة والمعدات وتنزيل أنظمة التشغيل والبرمجيات وكافة التطبيقات التي يحتاجها التشغيل وحسب متطلبات القوات المسلحة الأردنية ـ الجيش العربي عند الطلب.

2. يتم تسليم جميع المواد أعلاه في قيادة القوة البحرية والزوارق الملكية (العقبة).

3. تلتزم الشركة بوضع (label) على كل جهاز يبين (اسم الشركة المحال عليها العطاء، رقم قرار الإحالة، تاريخ التوريد، تاريخ انتهاء الكفالة).

4. أن تكون فترة الكفالة والصيانة المجانية لمدة لا تقل عن (24) شهرا تشمل جميع الأنظمة والبرمجيات والأجهزة والمعدات التي يتم توريدها.

5. تلتزم الشركة وخلال فترة الصيانة المجانية بتوفير القطع المستخدمة في صيانة الأجهزة والمعدات بحيث تكون معفاة من كافة الرسوم والضرائب والشحن والتخليص في حال استيرادها من الخارج بحيث يتم نقلها إلى موقع الجهاز على نفقة الشركة.

6. تلتزم الشركة وخلال فترة الصيانة المجانية بتوفير كافة التحديثات التي تطرأ على أنظمة التشغيل والبرمجيات الخاصة بالمعدات وعلى نفقة الشركة.

7.  تلتزم الشركة بتوفير قطع الصيانة والبرمجيات وأنظمة التشغيل الخاصة بالمعدات (مقابل الثمن أو من خلال عقود خاصة بالصيانة) لمدة لا تقل عن (60) شهر بعد انتهاء فترة الصيانة المجانية ولا تقل كفاءة عن المستخدمة بالنظام.

8. تلتزم الشركة بالاستجابة خلال (24) ساعة عند إبلاغها عن الأعطال من خلال الهاتف أو من خلال الإيميل الرسمي، وتلتزم الشركة بإصلاح الأعطال خلال (48) ساعة خلال فترة الصيانة المجانية وبتوفير أي جهاز او معدات أو برمجية تستلزم تشغيل النظام وعلى نفقة الشركة لحين إصلاح العطل.

9. تلتزم الشركة بتقديم وتسليم المشروع كحل متكامل لتشغيل نظام الصوتيات والمرئيات في القاعة التي سيعمل فيها النظام (turnkey solutions) وحسب المتطلبات الوظيفية.

10. تلتزم الشركة بأن تاريخ صنع الأجهزة والمعدات لا يزيد عن سنه شمسية من تاريخ الإحالة، وان تكون إصدارات البرمجيات الخاصة بالأنظمة حديثة وأصلية ويتم تحديثها بشكل دوري ومجانا من قبل الشركة.

11. ان لا تزيد مدة التوريد والتركيب والتسليم في العطاء عن (60) يوم عمل.

12. تقوم الشركة بعقد دورة داخلية لمنسبين عدد (5) من قيادة القوة البحرية والزوارق الملكية على استخدام وتشغيل النظام.

13. تلتزم الشركة بتقديم ورقة مطابقة (Compliance-Sheets) ومؤشر بخط فسفوري على Data-Sheets للمواصفات المطلوبة.

14. تلتزم الشركة بتسليم جميع المخططات (Riser & As-Built).

15. تلتزم الشركة بتسليم الارقام المتسلسلة للأجهزة التي سيتم تركيبها نسخة ورقية والكترونية.

16. تلتزم الشركة بتسليم جميع البرمجيات الخاصة بتشغيل النظام مع Source-Code)).

17. تلتزم الشركة بجميع الأعمال الإنشائية والكهربائية والمدنية(نجارة) لتركيب وتشغيل النظام.