- انواع وكميات الكوابل وصناديق التوزيع المطلوب شراؤها والشروط الخاصة بالشراء :

| # | المادة | الكمية |
| --- | --- | --- |
| 1 | كيبل غير مسلح (20) زوج بالمتر | 10000 |
| 2 | كيبل غير مسلح (100) زوج بالمتر | 8000 |
| 3 | كيبل غير مسلح (400) زوج بالمتر | 4000 |
| 4 | كيبل مسلح (40) زوج بالمتر | 10000 |
| 5 | كيبل مسلح (300) زوج بالمتر | 4000 |
| 6 | كيبل كواكسل بالمتر | 10000 |
| 7 | صندوق توزيع 80 خط بالحبة | 30 |
| 8 | صندوق توزيع 40 خط بالحبة | 30 |
| 9 | كيبل (د10) بالبكرة | 300 |

- المواصفات للكوابل وصناديق التوزيع اعلاه:

| مواصفات الكوابل المسلحة |  |  |
| --- | --- | --- |
| 1 | Conductor | Solid annealed copper having the diameter of 0.5 mm |
| 2 | Insulation | Foam skin insulation must be available |
| 3 | Cable Assembly | twisted pairs, each having special lay length to minimize the crosstalk and capacitance unbalance |
| 4 | Filling Compound | The water resistant filling compound is applied to the air spaces within the cable core to provide the water-proof. التأكيد على نوعية المادة العازلة اللزجة ( الشحمه ) ان تكون من مواصفات عالية اللزوجة |
| 5 | Core Covering | A non-hygroscopic dielectric plastic tape having suitable overlap is applied longitudinally or helically over the cable core |
| 6 | Flooding Compound | Sufficient amount of flooding compound shall be applied between the core covering material and screen |
| 7 | Screen (Shield) | A flat aluminum foil coated with copolymer on both sides applied longitudinally over the core covering as screen |
| 8 | Inner Jacket (Inner Sheath) | Black linear-low density or medium density polyethylene compound is extruded over the screen. |
| 9 | Armor | Corrugated steel tapes having a suitable overlap are applied longitudinally over the inner jacket to improve the mechanical strength of the cable. Special type of water-proof tape or flooding compound shall be applied the interfaces between the armor and both jackets (inner and outer |
| 10 | Outer Jacket (Sheath) | Black linear-low density or medium- density polyethylene compound is extruded over the armor |
| 11 | Identification Tape | A suitable tape, durably marked with the manufacturer's name, year of manufacture and type of cable |
| 12 | Length Marking | sequentially numbered length markings are located at alternate 1 meter intervals on the outside of the jacket |
| 13 | packing | 500 m/ wooden drum |

| مواصفات الكوابل غير المسلحة |  |  |
| --- | --- | --- |
| 1 | Conductor | Solid annealed copper having the diameter of 0.5 mm |
| 2 | Insulation | Foam skin insulation must be available |
| 3 | Cable Assembly | twisted pairs, each having special lay length to minimize the crosstalk and capacitance unbalance |
| 4 | Filling Compound | The water resistant filling compound is applied to the air spaces within the cable core to provide the water-proof. التأكيد على نوعية المادة العازلة اللزجة ( الشحمه ) ان تكون من مواصفات عالية اللزوجة |
| 5 | Core Covering | A non-hygroscopic dielectric plastic tape having suitable overlap is applied longitudinally or helically over the cable core |
| 6 | Flooding Compound | Sufficient flooding compound must be applied between the core covering material and screen |
| 7 | Screen (Shield) | Coated with polymer on both sides, flat and having aluminum foil is applied longitudinally over the core covering as screen |
| 8 | Jacket (Sheath) | Black linear-low density or medium density polyethylene compound is extruded over the screen |
| 9 | Identification Tape | A suitable tape, durably marked with the manufacturer's name, year of manufacture and type of cable |
| 10 | Length Marking | sequentially numbered length markings are located at alternate 1 meter intervals on the outside of the jacket |
| 11 | packing | 500 m/ wooden drum |
| 12 | anti scratch |  |

مواصفات كيبل كواكسل

Coaxial Cable 75 ohm (BT3002) / 1*0.31mm sold core,pvc jaket 100 متر لكل بكرة

| مواصفات كيبل (د10) |  |  |
| --- | --- | --- |
| المواصفات | ملاحظات |
| Conductor Construction | : 7x0.3mm= 0.5mm2 .= (3g.Steel+4 T.CU) x0.3mm. (Wires Stranding for two conductors one is L.H and other R.H) |
| Insulation Thickness | 0.43 mm – BK- P.E(Polyethylene) |
| Two Conductors | are run parallel and insulated with BK-P.E with groove in between the conductors for easy separation |
| Diameter over all | 1.8x3.6mm |
| Cable Weight | Min 12.5 Kg/km |
| Wooden Reel Drum length | 500 meters |
| Braking load for two conductors | (170) LBS Minimum |
| Insulation Resistance | 3500 M OHM / Km at 20 C |
| Conductors Loop Resistance | two Conductors = 120 ohm /Km at 20 C |
| 2x0.5mm Field Telephone Wire |  |

- مواصفات صناديق التوزيع :

1.  فتحة الكوابل من اسفل الصندوق ومزودة بشبر مطاط يمنع تجرح الكوابل.

2.  العلب مصنوعة من صاج مجلفن ومطلي بطلاء عازل وحراري.

3.  ابواب العلب مزودة بشبر مطاطي يمنع دخول الماء للصندوق.

4.  الستربات بنظام الكبس ومصنوعة من مادة مقاومة للصدأ.

- الشروط الخاصة لشراء كوابل مختلفة وصناديق التوزيع :

# 1.   يجب على الشركة بيان بلد المنشأ للمواد المعروضة.

2.   يجب ان تكون المواد جديدة (100%) ومن انتاج عام 2025.

3.  تلتزم الشركة المناقصة بتقديم العرض كما هو مبين بالملحق الخاص بالمواصفات الفنية المرفق بوضوح تام وكما هو مطلوب بدعوة العطاء.

4.  إمكانية تقديم عينة في حال تم طلبها من لجنة الشراء للتاكد من جودتها.

5.  يمكن تجزئة العطاء على اكثر من شركة.

6.  الاسعار على اساس واصل مستودعات اللاسلكي الرئيسية/الزرقاء – وادي الحجر.

7.  يسمح بتفاوت طول الكيبل على كل بكرة او لفة بنسبة ( 2% ) زيادة او نقصان.

8.  في حال كانت الشركة تقدمت بعينات خلال عملية طرح العطاء سابقاً ورغبتها بالمشاركة بمواد من نفس المصدر وإعتماد نتيجة تقييم هذه العينات في عملية تقييم العرض الجديد عليها ذكر بشكل واضح في العرض الفني والمالي المقدم من قبلها وبعكس ذلك عليها تقديم عينه من المواد المعروضة في الموعد المحدد لتسليم العروض.