Item (1): CAMERAS

(1.1) Outdoor IP Camera              		                          QTY (36)

| Minimum requirements |  |  |
| --- | --- | --- |
| No. | specs | Description |
| 1 | Resolution | 5Megapixel progressive CMOS · H 265+ , H.265 & H.264 dual-stream encoding |
| 2 | Video streaming | 20fps@ 5Megapixel |
| 3 | WDR | WDR (120dB), Day/Night(ICR), 3DNR, AWB, AGC, BLC |
| 4 | Lens | 1/3” 2.7~13.5mm varifocal lens |
| 5 | Memory | Micro SD card slot, up to 128GB |
| 6 | IR | IR LEDs Length 50 m |
| 7 | Power | PoE |
| 8 | Defog | built in defog. |
| 9 | Weather protection | Weatherproof: IP67 |
| 10 | Certificate | CE and FCC certificate |
| 11 | Warranty | All Items shall be covered with three Years mother company warranty and support |
| 12 | Installation | Bidder should provide full installation and configuration for all proposed components. Bidder is responsible for test and insuring that all services are up, running and work without any issues. |

(1.2)  Network Video Recorder  64 Ch   		       QTY (1)

| Minimum requirements |  |  |
| --- | --- | --- |
| No. | Specs | Description |
| 1 | Processor | Intel Processor (High performance) |
| 2 | Number of channels | 64 IP Camera Inputs |
| 3 | Bandwidth | 320 Mbps Incoming Bandwidth |
| 4 | Preview and Playback Resolution | Up to 12MP Resolution for Preview and Playback |
| 5 | Intelligent Video specs | Smart Tracking, ANR (Automatic Network Replenishment Technology) and Intelligent Video |
| 6 | Warping | Fisheye DE warping |
| 7 | 4K Resolution | Support 4K Resolution |
| 8 | RAID | Support RAID 0, 1,5, 6, 10 |
| 9 | Video System | Intelligent Video System (IVS) |
| 10 | Client management software | Mandatory |
| 11 | Warranty | All Items shall be covered with three Years mother company warranty and support |
| 12 | Installation | Bidder should provide full installation and configuration for all proposed components. Bidder is responsible for test and insuring that all services are up, running and work without any issues. |

(1.3) Cabinet                                               		       QTY (4)

| Minimum requirements |  |  |
| --- | --- | --- |
| No. | specs | Description |
| 1 | Cabinet | Waterproof outdoor cabinet for switches and fiber joints |
| 2 | Installation | Bidder should provide full installation and configuration for all proposed components. Bidder is responsible for test and insuring that all services are up, running and work without any issues. |

(1.4) Fiber SM installation                                     	       QTY (750 m)

| Minimum requirements |  |  |
| --- | --- | --- |
| No. | specs | Description |
| 1 | Fiber cable | The fiber should be single-mode for outdoor purpose |
| 2 | Cores No . | 12 core |
| 3 | Fiber protection | The fiber should be installed in external PVC pipes |
| 4 | Installation | Bidder should provide full installation and configuration for all proposed components. Bidder is responsible for test and insuring that all services are up, running and work without any issues. All UTP or fiber cables , patch cords, and big-tails needed for install and run the system should be included |

(1.5) 16 Ports POE Access Switches                              Qty (6)

| Minimum requirements |  |  |
| --- | --- | --- |
| No. | specs | Description |
| 1 | Ports | 16 x 10/100/1000 Mbps RJ45 Ports. (POE+) |
| 2 | SFP Ports | 2 SFP 1G, all ports should be active. |
| 3 | Stack ability | Support the ability to manage at least 8 interconnected switches as a single logical device in a form of virtual chassis system |
| 4 | Manage | Should be fully managed switch |
| 5 | Non-blocking | Non-blocking architecture. |
| 6 | Throughput | 18 Gbps forwarding , 36 Gbps switching |
| 7 | POE power | Power over Ethernet power budget: min 120 Watt |
| 8 | Power supply | include a single fixed power supply |
| 9 | Cables | All needed fiber batch cord shall be provided |
| 10 | Bluetooth | The switches support an external Bluetooth dongle that plugs into the USB port on the switch and allows a Bluetooth-based RF connection with external laptops and tablets |
| 11 | Warranty | All Items shall be covered with Three Years mother company warranty and support |

(1.6)    HDD                                                                      Qty (6)

| Minimum requirements |  |  |
| --- | --- | --- |
| No. | specs | Description |
| 1 | Capacity | 6 TB |
| 2 | Used type | specifically for mainstream surveillance systems |
| 3 | Warranty | Three Years mother company warranty and support |

(1.7) Transceivers (SFP)

| Minimum requirements |  |  |  |
| --- | --- | --- | --- |
| No. | Specs | Description |  |
| 1 | Single -mode transceiver | 1 G SR | QTY ( 10 ) |

Item (2): Computers, laptops and Printers :

(2.1)    PC (business computer system)                                       Qty (25)

| Minimum requirements |  |  |
| --- | --- | --- |
| No. | Specs | Description |
| 1 | Operating system | Free DOS |
| 2 | Product series | Business series |
| 3 | Form Factor | Micro tower or tower |
| 4 | Processor | Intel® Core™ i7 12th generation (2 GHz Up To 4.5Ghz ,25MB cache, 12 Core) |
| 5 | Chipset | Intel |
| 6 | Memory | 8GB DDR4 - 3200 (1x 8 GB) RAM up to 32 GB |
| 7 | Hard Drive | 500 GB M.2 SSD |
| 8 | Video | Integrated UHD Graphics card HDMI port , Display port |
| 9 | Audio | Integrated Audio card Stereo Headphone/Microphone combo jack |
| 10 | Network | WLAN 802.11 a/g/n Integrated 10/100/1000 GbE LAN |
| 11 | Power | 220 volt 50Hz Power Supply with UK Plug |
| 12 | Keyboard | Arabic & English imprinted USB Keyboard should be the same brand of PC |
| 13 | Mouse | USB optical Scroll Mouse with pad should be the same brand of PC |
| 14 | USB Ports | 6 USB (2 USB 3.2 +4 USB 2.0),1 RJ-45 ,1 HDMI ,1 DP, 1 Audio in ,1 Audio out |
| 15 | Optical drive | Should be included in the Chassis. |
| 16 | Display | 21" LED Monitor with similar input to the proposed Graphics output. |
| 17 | Brand/Country of origin | USA, Europe, or Japan Only Or, Company is one of the top 5 PC Market share leaders in 2024. |
| 18 | Warranty | three Years |
| 19 | Installation Services | Hardware and Software Onsite installation service |
| 20 | Others | System Unit, Chassis, Motherboard, Monitor, Mouse and Keyboard are Same Brand |

(2.2)    PC (business Laptop system)                                    Qty (4)

| Minimum requirements |  |  |
| --- | --- | --- |
| No. | specs | Description |
| 1 | Operating system | Free DOS |
| 2 | Product series | Normal series |
| 3 | Form Factor | laptop |
| 4 | Processor | Intel® Core™ i7 12th generation (1 GHz Up To 4Ghz ,8MB cache, 8 Core) |
| 5 | Chipset | Intel |
| 6 | Memory | 8GB DDR4 - |
| 7 | Hard Drive | 500 GB M.SSD m2 |
| 8 | Video | HDMI port , Display port |
| 9 | Audio | Integrated Audio card Stereo Headphone/Microphone combo jack |
| 10 | Network | WLAN 802.11 a/g/n Integrated 10/100/1000 GbE LAN |
| 11 | Power | 11 Volt battery with 45 hour |
| 12 | Keyboard | Arabic & English imprinted USB Keyboard |
| 13 | Pointer device | Touchpad |
| 14 | USB Ports | 3USB (2 USB 3.2 +1 USB 2.0),1 RJ-45 ,1 HDMI or 1 DP, 1 Audio in ,1 Audio out |
| 15 | Optical drive | Should be included in the Chassis. |
| 16 | Display | 15 inch |
| 17 | Brand/Country of origin | USA, Europe, or Japan Only Or, Company is one of the top 5 PC Market share leaders in 2024. |
| 18 | Warranty | three Years |
| 19 | Installation Services | Hardware and Software Onsite installation service |
| 20 | Others | System Unit, Motherboard, display are Same Brand |

(2.3)    A4 Laser printer                                                  Qty (15)

| Minimum requirements |  |  |
| --- | --- | --- |
| No. | specs | Description |
| 1 | Print Speed | 33 PPM |
| 2 | Resolution | 1200x1200 dpi |
| 3 | Memory (RAM) | 128 MB Memory |
| 4 | Connectivity | USB 2.0 |
| 5 | Warranty | Three years warranty including Labor and spare parts |
| 6 | Others | All Drivers and Utilities should be included, USB Cable |

Item (3): Switches and Routers (all switches and cores should be the same brand in this item):

(3.1)    core switches                                                            Qty (2)

| Minimum requirements |  |  |
| --- | --- | --- |
| No. | Specs | Description |
| 1 | Ports | 24-ports 25G/10G/1G , fixed 1 RU switch |
| 2 | Capacity | Up to 2 Tbps switching capacity |
| 3 | Forwarding rate | Up to 1 Bpps of forwarding rate |
| 4 | MAC table | MAC address table minimum 64,000 |
| 5 | RAM | DRAM and Flash minimum 16GB |
| 6 | Protocol | Supports BGP, EIGRP, HSRP, IS-IS, BSR, MSDP, PIM SM, PIM SSM, PIM-BIDIR, IP SLA, OSPF,Macsec-128 , CDP |
| 7 | Features | SDN capability for future use |
| 8 | Power supply | Dual power supplies |
| 9 | Fan | Field replaceable fans |
| 10 | Layer | Layer 3 switch |
| 11 | SW Level | Enterprise level switches |
| 12 | Vendor | Vendor shall be a leader based on Gartner in Wired and Wireless LAN for the last five years |
| 13 | Partnership | The bidder should be an official partner with the vendor with highest level of partnership , and should be at least 5 Gold level parnters in Jordan (highest) |
| 14 | Warranty | 3 years mother company warranty with next business day replacement at RMS site, Vendor must have local Depot in Jordan. |
| 15 | Devices name | Devices should be under RMS name and bidder should provide contract number with Vendor that contains all devices serials covered as an evidence |
| 16 | Installation | Bidder should provide full installation and configuration for all proposed components. Bidder is responsible for test and insuring that all services are up, running and work without any issues. |

(3.2)  Access Switches Stackable 24-Port with 10G Uplinks      Qty (8)

| Minimum requirements |  |  |
| --- | --- | --- |
| No. | Specs | Description |
| 1 | SW Level | Switches should be 1U rack mounted form factor, Enterprise level |
| 2 | Ports | Should have 24x ports 10/100/1000BASE-T Ports & 4x 1G/10G SFP+ ports |
| 3 | Forwarding rate | Should support high performance switching with minimum 128 Gbps |
| 4 | Throughput | Should support high performance throughput with minimum 95 Mpps |
| 5 | Stack ability | Stacking with dedicated ports/cables with up tp 8 switches per stack |
| 6 | Layer | Layer 2, Static Routing, Routed Access, PBR, PIM Stub Multicast (1000 routes), PVLAN, VRRP, PBR, CDP |
| 7 | Vlans | VLAN Supported: 4K |
| 8 | MAC table | MAC address table size at least 16K |
| 9 | Features | Should Support MacSec Should Support SDN for future use |
| 10 | Vendor | Vendor shall be a leader based on Gartner in Wired and Wireless LAN for the last five years |
| 11 | Partnership | The bidder should be an official partner with the vendor with highest level of partnership , and should be at least 5 Gold level partners in Jordan (highest) |
| 12 | warranty | 3 years mother company warranty with next business day replacement at RMS site , Vendor must have local Depot in Jordan. |
| 13 | Devices name | Devices should be under RMS name and bidder should provide contract number with Vendor that contains all devices serials covered as an evidence |
| 14 | Installation | Bidder should provide full installation and configuration for all proposed components. Bidder is responsible for test and insuring that all services are up, running and work without any issues. |

(3.3)  Access Switches Stackable 48-Port with 10G Uplinks      Qty (9)

| Minimum requirements |  |  |
| --- | --- | --- |
| No. | specs | Description |
| 1 | SW Level | Switches should be 1U rack mounted form factor, Enterprise level |
| 2 | Ports | Should have 48x ports 10/100/1000BASE-T Ports 4x 1G/10G SFP ports |
| 3 | Forwarding rate | Should support high performance switching with minimum 176 Gbps |
| 4 | Throughput | Should support high performance throughput with minimum 130 Mpps |
| 5 | Stack ability | Stacking with dedicated ports/cables with up tp 8 switches per stack |
| 6 | Layer | Layer 2, Static Routing, Routed Access, PBR, PIM Stub Multicast (1000 routes), PVLAN, VRRP, PBR, CDP |
| 7 | Vlans | VLAN Supported: 4K |
| 8 | MAC table | MAC address table 16K |
| 9 | Features | Should Support MacSec Should Support SDN for future use |
| 10 | Vendor | Vendor shall be a leader based on Gartner in Wired and Wireless LAN for the last five years |
| 11 | Partnership | The bidder should be an official partner with the vendor with highest level of partnership , and should be at least 5 Gold level partners in Jordan (highest) |
| 12 | warranty | 3 years mother company warranty with next business day replacement at RMS site , Vendor must have local Depot in Jordan. |
| 13 | Devices name | Devices should be under RMS name and bidder should provide contract number with Vendor that contains all devices serials covered as an evidence |
| 14 | Installation | Bidder should provide full installation and configuration for all proposed components. Bidder is responsible for test and insuring that all services are up, running and work without any issues. |

(3.4)  Access Switches 24-Port Full POE for CCTV/VoIP      Qty (12)

| Minimum requirements |  |  |
| --- | --- | --- |
| No. | specs | Description |
| 1 | Ports | 24-port 1G POE+ ports with 4x 10G SFP |
| 2 | PoE | POE+ with power budget of 375W |
| 3 | Throughput | up to 128 Gbps switching capacity |
| 4 | Forwarding rate | up to 95 Mpps of forwarding rate |
| 5 | MAC table | Mac address table min 16,000 |
| 6 | Frame size | Jumbo frame 9000 bytes |
| 7 | Features | Layer 3 capabilities |
| 8 | Vendor | Vendor shall be a leader based on Gartner in Wired and Wireless LAN for the last five years |
| 9 | Partnership | The bidder should be an official partner with the vendor with highest level of partnership , and should be at least 5 Gold level partners in Jordan (highest) |
| 10 | warranty | 3 years mother company warranty with next business day replacement at RMS site , and devices should be under RMS name and bidder should provide contract number with Vendor that contains all devices serials covered |
| 11 | Devices name | Devices should be under RMS name and bidder should provide contract number with Vendor that contains all devices serials covered as an evidence |
| 12 | Installation | Bidder should provide full installation and configuration for all proposed components. Bidder is responsible for test and insuring that all services are up, running and work without any issues. |

(3.5)  Core Switch for CCTV/VoIP                                          Qty (1)

| Minimum requirements |  |  |
| --- | --- | --- |
| No. | specs | Description |
| 1 | Ports | 24-port 10G SFP+ ports |
| 2 | PoE | POE+ with power budget of 375W |
| 3 | Throughput | up to 480 Gbps switching capacity |
| 4 | Forwarding rate | up to 357 Mpps of forwarding rate |
| 5 | MAC table | Mac address table min 16,000 |
| 6 | Frame size | Jumbo frame 9000 bytes |
| 7 | Features | Layer 3 capabilities |
| 8 | Vendor | Vendor shall be a leader based on Gartner in Wired and Wireless LAN for the last five years |
| 9 | Partnership | The bidder should be an official partner with the vendor with highest level of partnership , and should be at least 5 Gold level partners in Jordan (highest) |
| 10 | warranty | 3 years mother company warranty with next business day replacement at RMS site , and devices should be under RMS name and bidder should provide contract number with Vendor that contains all devices serials covered |
| 11 | Devices name | Devices should be under RMS name and bidder should provide contract number with Vendor that contains all devices serials covered as an evidence |
| 12 | Installation | Bidder should provide full installation and configuration for all proposed components. Bidder is responsible for test and insuring that all services are up, running and work without any issues. |

(3.6)  Transceivers (SFP)

| Minimum requirements |  |  |  |
| --- | --- | --- | --- |
| No. | specs | Description |  |
| 1 | Multi-mode transceiver | 10 G SR | QTY ( 34 ) |
| 2 | Single-mode transceiver | 10 G LR | QTY ( 24 ) |

(3.7)  Router with 2 X E1 Interfaces                                        Qty (1)

| Minimum requirements |  |  |
| --- | --- | --- |
| No. | specs | Description |
| 1 | Ports | 4 X1G WAN ports (2 X SFP , 2 X Copper) |
| 2 | Throughput | IPsec throughput up to 1Gbps |
| 3 | Compatibility | It must be from the same vendor of the switches |
| 4 | E1 card | 2 X E1 ports NIM card |
| 5 | Vendor | Vendor shall be a leader based on Gartner in Wired and Wireless LAN for the last five years |
| 6 | Partnership | The bidder should be an official partner with the vendor with highest level of partnership , and should be at least 5 Gold level partners in Jordan (highest) |
| 7 | warranty | 3 years mother company warranty with next business day replacement at RMS site , and devices should be under RMS name and bidder should provide contract number with Vendor that contains all devices serials covered |
| 8 | Devices name | Devices should be under RMS name and bidder should provide contract number with Vendor that contains all devices serials covered as an evidence |
| 9 | Installation | Bidder should provide full installation and configuration for all proposed components. Bidder is responsible for test and insuring that all services are up, running and work without any issues. |

Item (4): Training

(4.1): Network Security and Management Course       	           	QTY(2)

| Course Name | Network Security and Management Course |
| --- | --- |
| Training Center | Certified Training Center in UK , USA or Dubai |
| Trainer & Material | Certified Trainer & Original Material |
| Expenses | All expenses (course Fees, Visa's, Airplane Tickets, Accommodation , Transportation, etc.) |

الشروط الخاصة :

- يجب ان تكون كافة المواد المعروضة جديدة 100%   وان يكون تـاريخ الصنع ليس قبل عام 2024.

- يجب اتن تكون الاسعار بالدينار الاردني  معفاة من كافة الرسوم الجمركيه والضريبه العامه على المبيعات واية رسوم اوعوائد اخرى تسمح القوانين والانظمة باعفاء مشتريات القوات المسلحة الاردنية-الجيش العربي منها.

- يجب على الشركة بيان بلد المنشأ للأجهزة المعروضة.

- تلتزم الشركة بتزويد القوات المسلحة الاردنية بقطع الغيار اللازمة ولمدة خمس سنوات,  بعد انتهاء الكفالة المجانية , مقابل الثمن حسب الاسعار السائدة  في حينه.

- على الشركة توضيح كل مواصفة من مواصفات العطاء بعرضها وارفاق ما يثبت ( كتالوجات وعرض الشركة ) بأن المواصفة متوفرة بالاجهزة المقدمة و تقديم Compliance Sheet مع ذكر رقم الصفحة بالكتالوج والعرض , مقابل كل مواصفة.

- يجب بيان نوع وموديل الجهاز المقدم .

- توريد وتركيب وتشغيل المواد كافة في الخدمات الطبية الملكية في الموقع الذي تقرره شعبة تكنولوجيا المعلومات الطبية .

- تلتزم الشركة المحال عليها العطاء بتنزيل كافة البرمجيات المطلوبة من ضمن العطاء .

- يجب ان تكون الاجهزة وكافة تفرعاتها اصلية BRAND NAME وصالحة  .

- الكفالة المجانية لمدة 3 سنوات  من تاريخ التسليم على جميع المواد الا اذا ذكر كفالة لمدة أعلى في بند المواصفات لكل جهاز .

- وضع ليبل يوضح  به ( اسم الشركة , وتاريخ الإحالة ,الكفالة , ورقم هاتف معتمد لطلب الصيانة ) ويجب ان يكون ليبل مثبت ولاصق بشكل جيد على الاجهزه .