Digital Mobile Radio (Portable)

Technical Specifications

| General |  |
| --- | --- |
| Version | Latest version |
| Frequency range | VHF: (136 – 174) MHz |
| Operating modes | Tier II: in accordance to ETSI standards TS 102 361_1 TS 102 361_2 TS 102 361_3 Analogue operation compatible with conventional radios. |
| Channel capacity | >500 |
| Channel spacing | 12.5 kHz , 25 kHz |
| Antenna impedance | 50Ω |
| Battery capacity | ≥ 2000Mah |
| Display | LCD Alphanumeric full colored ≥4 lines |
| GPS | Embedded GPS |
| General Features | Group call, Individual call, , All call, Short Data Service, Authentication, Caller ID, Call History, Contact List (phone book), Multiple language capability(Arabic preferably) Time Display, Keypad (letters, numerical), Keypad Lock. |
| Encryption | AES 256 |
| Interoperability | Fully interoperable with any other DMR manufactures including encryption mode. |

Digital Mobile Radio (Portable)

Technical Specifications

| ENVIRONMENTAL SPECIFICATIONS |  |
| --- | --- |
| Operating Temperature | - 30 °C to + 60 °C or better |
| Storage Temperature | - 40 °C to + 80 °C or better |
| IP rating | ≥ IP 67 |
| Ruggedness | Comply to MIL-STD-810 D / E / F / G |

| Transmitter |  |
| --- | --- |
| Transmitting power | VHF :≥4W (adjustable) |
| Frequency Stability | ±0.5ppm or better |
| Modulation | FM 4FSK digital modulation |
| Nominal audio distortion | ≤ 3% |
| Digital vocoder type | AMBE+2 |

| Receiver |  |
| --- | --- |
| Sensitivity (Analogue) | ≤ -117 dbm @12 db SINAD |
| Sensitivity (Digital) | ≤ -117 dbm @ BER 5% |
| Adjacent channel selectivity | ETSI : 60 Db at 12.5 kHz / 70 Db at 25 kHz |
| Intermodulation | ETSI: 65 Db at 12.5 / 25 Kh |
| Spurious response rejection | ETSI: 70 Db at 12.5 / 25 kHz |
| Nominal audio distortion | ≤ 3% |

Digital Mobile Radio (Portable)

Technical Specifications

NOTES

1.	Each radio set should include:

a.    One rechargeable battery.

b.    One AC Charger.

c. Full band (136-174) MHz antenna sub band not accepted, GPS integrated.

d.    One belt clip.

2.	Fully featured DMR Tier2 minimum upgradable to Tier3, where breakdown prices of upgrading to Tier3 (HW, SW and licenses) shall be provided. Bidders shall adhere to these prices for the next (5) five coming years by delivering a certificate showing this condition.

3.	Two years warranty min.

4.	Ability to operate with conventional mode.

Digital Mobile Radio (Mobile)

Technical Specifications

| General |  |
| --- | --- |
| Frequency range | VHF: (136 – 174) MHz |
| Operating modes | Tier II: in accordance to ETSI standards TS 102 361_1 TS 102 361_2 TS 102 361_3 Analogue operation compatible with conventional radios |
| Channel capacity | >500 |
| Channel spacing | 12.5 kHz , 25 kHz |
| Power Supply | 12VDC Nominal |
| Antenna impedance | 50Ω |
| Display | LCD Alphanumeric full colored ≥ 4 lines |
| GPS | Embedded GPS |
| General Features | Group call, Individual call, , All call, Short Data Service, Authentication, Caller ID, Call History, Contact List (phone book), Multiple language capability(Arabic preferably) Time Display, Keypad (letters, numerical), Keypad Lock. |
| Encryption | AES 256 |
| Interoperability | Fully interoperable with any other DMR manufactures including encryption mode. |

Digital Mobile Radio (Mobile)

Technical Specifications

| ENVIRONMENTAL SPECIFICATIONS |  |
| --- | --- |
| Operating Temperature | - 30 °C to + 60 °C or better |
| Storage Temperature | - 40 °C to + 80 °C or better |
| IP rating | ≥ IP 67 |
| Ruggedness | Comply to MIL-STD-810 D / E / F / G |

| Transmitter |  |
| --- | --- |
| Transmitting power | VHF ≥ 45W (adjustable) |
| Frequency Stability | ±0.5 ppm or better |
| Modulation | FM 4FSK digital modulation |
| Nominal audio distortion | ≤ 3% |
| Digital vocoder type | AMBE+2 |

| Receiver |  |
| --- | --- |
| Sensitivity (Analogue) | ≤ -117 dbm @12 db SINAD |
| Sensitivity (Digital) | ≤ -117 dbm @ BER 5% |
| Adjacent channel selectivity | ETSI : 60 dB at 12.5 kHz / 70 dB at 25 kHz |
| Intermodulation | ETSI: 65 dB at 12.5 / 25 kHz |
| Spurious response rejection | ETSI: 70 dB at 12.5 / 25 kHz |
| Nominal audio distortion | ≤ 3% |

Digital Mobile Radio (Mobile)

Technical Specifications

NOTES:

1. 	Each radio set should include:

a.   One Handheld microphone.

b. One power Supply with cables (input volt 220V AC 50/60 Hz / output volt (12-15) VDC / output current 30A min).

c. Full band (136-174) MHz antenna sub band not accepted.

d. GPS magnetic antenna (preferable to be integrated with VHF antenna).

2. 	Availability of the specification pointed as "Preferably gives an advantage if companies are equal in the rest of all other specifications.

3.	Fully featured DMR Tier2 minimum upgradable to Tier3, where breakdown prices of upgrading to Tier3 (HW, SW and licenses) shall be provided. Bidders shall adhere to these prices for the next (5) five coming years by delivering a certificate showing this condition.

4.	Two years warranty min.

Quantities & Accessories

| # | Item | QTY |
| --- | --- | --- |
| 1 | Digital Mobile Radio (Portable) / latest version | 400 |
| 2 | Spare charger and Spare VHF antenna for radio above | 100 |
| 3 | Digital Mobile Radio (Mobile) | 200 |
| 9 | Programming Hardware & Software Kits (for Portable Radio) with necessary cables | 10 |
| 10 | Programming Hardware &Software (for Mobile Station Radio ) with necessary cables | 10 |
| 11 | Maintenance Training for 5 working days (Covering All: Travelling Tickets, Accommodations, Transportations, Medical Insurance, etc…) | (7) Persons |

الشروط الخاصة:

- على الشركة بيان مدة التوريد بشكل واضح في العرض الفني والمالي وعلى ان لا تزيد عن (6) أشهر من تاريخ التبليغ.

- على الشركة بيان بلد المنشأ للمواد المعروضة وتلتزم الشركة المحال عليها اللوازم مدار بحث هذا العطاء بتقديم شهادة منشأ صادرة ومصدقة من الغرفة التجارية في بلد المنشأ.

- يجب ان تكون المواد المعروضة صالحة وجديد 100% وغير مجددة ومن انتاج ليس قبل النصف الثاني لعام 2024.

- تلتزم الشركة المحال عليها اللوازم بتقديم كفالة صيانة للوازم الموردة لمدة لا تقل عن (24) شهر من تاريخ الاستلام النهائي للوازم وقبولها.

- تضمن الشركة المحال عليها اللوازم خلو اللوازم المحاله من سوء المصنعية لمدة لا تقل عن (12) شهراً من تاريخ الاستلام النهائي للوازم وقبولها.

- يجب ان تكون الاسعار بالدينار الاردني معفاة من كافة الرسوم الجمركية ومن الضريبة العامة على المبيعات واية رسوم او ضرائب تسمح القوانين والانظمة باعفاء القوات المسلحة الاردنية –الجيش العربي منها.

- يجب ان يتم تسعير المواد على اساس التوريد لمستودعات القوات المسلحة الاردنية –الجيش العربي/الزرقاء –وادي الحجر.

- يجب ان يكون سعر كل مادة من المواد المعروضة متضمن اجور الشحن والنقل واستصدار الموافقات اللازمة والشهادات المطلوبة واية مصاريف اخرى لحين وصولها الى مستودعات القوات المسلحة                   الاردنية-الجيش العربي ويحق للجنة الشراء استبعاد اي عرض مالي يتم تسعير هذه النفقات فيه كبند منفصل.

- في حال توريد اللوازم المحالة من خارج المملكة فيتم الشحن الى مطار الملكة علياء الدولي او ميناء العقبة فقط ويتم نقلها الى قيادة مستودعات اللاسلكي الرئيسية من قبل الشركة وعلى نفقتها ويحق للقوات المسلحة الاردنية-الجيش العربي رفض استلام اية مادة يتم شحنها بطريقة مخالفة وتغريم الشركة بدل العطل والضرر الذي لحق نتيجة ذلك واية غرامات اخرى.

- العطاء قابل للتجزئة على أكثر من شركة.

- لن يتم النظر في اي عرض لا يكون مبين فيه جميع المواصفات الفنية المطلوبة ومدى التزام الشركة بجميع الشروط.

- على الشركة ارفاق كشف كميات بالمواد المعروضة في العرض الفني بالإضافة الى كشف مطابقة فنية ممع مراعاة عدم ذكر اية تفصيلات مالية في العرض الفني.

- على الشركة ارفاق كافة النشرات الفنية والوثائق التي تثبت مطابقة المواد المعروضة لما هو مطلوب في العطاء مع تأشير المواصفات الفنية المطلوبة بدعوة العطاء عليها.

- يجب ان يكون العرض المقدم ساري المفعول لمدة لا تقل عن (120) يوم من تاريخ اغلاق العطاء وبعكس ذلك يتم اهمال العرض.

- طريقة الدفع: بعد الاستلام النهائي.

- تلتزم الشركة المحال عليها اللوازم بالاستبدال الفوري للوازم المعطلة في حال سوء المصنعية.

- تلتزم الشركة بتزويد قطع احتياطية ولمدة لا تقل عن (10) سنوات من تاريخ التوريد.

- تلتزم الشركات بتقديم عينة لأي جهاز معروض لم يستخدم مسبقاً في القوات المسلحة الأردنية وذلك لإجراء فحص موائمة مع الأجهزة العاملة في القوات المسلحة الأردنية وسيتم اعتماد فحص الموائمة في عملية التقييم الفني للعروض.

- تلتزم الشركة المحال عليها العطاء بتسليم نسخة (CD) و كشوفات ورقية تبين (S.N) لجميع الأجهزة التي يتم توريدها الى مستودعاتنا عند توريد المواد.

- يتم تقديم العروض ورقياً وتلتزم الشركات بتقديم نسخه الكترونية من العرض الفني في مغلف العرض الفني ونسخه الكترونية من العرض المالي في مغلف العرض المالي.